Yes. Even without a diagnostic tool, you can do a fairly good practical battery test using only the M7’s own displays plus the charger’s energy reading. You just won’t be able to see true cell-level SOH or individual cell voltages.

For the current Geely Galaxy M7 225 km version, Geely specifies an LFP battery, 225 km CLTC EV range, and 30–80% fast charging in about 15 minutes under test conditions. Galaxy Geely

Here is the test I would do before buying it:

- Start around 95–100% SOC. Reset the trip meter and average energy consumption. Take photos of:

  - battery %

  - estimated EV range

  - odometer

  - average kWh/100 km

- Select pure EV mode and drive normally. For your kind of city driving, ideally test at least 80–120 km, and better yet continue until the battery reaches roughly 10–20%. Keep the gasoline engine from intentionally running unless the vehicle overrides EV mode for protection.

- Record:

  - kilometers driven

  - starting and ending SOC

  - average kWh/100 km

  - remaining indicated EV range

For example, suppose you start at 100%, stop at 20%, drive 135 km and the screen says 16 kWh/100 km. Your approximate electricity consumption was:

135 × 16 / 100 = 21.6 kWh

Since that used about 80% of the battery's usable SOC window, that would be quite believable for a roughly 30-kWh-class battery.

### A very useful charging test

After driving it down to around 10–20%, plug it into a charger that displays kWh delivered.

Write down:

SOC before charging → SOC after charging → kWh taken from charger

For example:

15% → 100%
charger supplied 27 kWh

That looks plausible because charger energy includes losses.

But something like:

15% → 100%
charger supplied only 14 kWh

on an M7 with a ~30-kWh-class battery would deserve investigation.

### Check whether it loses charge while parked

After fully charging, park it overnight without using it.

If you leave it at:

100% tonight

and tomorrow morning it shows:

98–100%, that's generally fine.

Even a small few-percent adjustment can occur because SOC estimation changes with battery temperature and balancing.

But:

100% → 85% overnight

without climate control or other heavy loads would be suspicious.

### Check the battery-range behavior

Don't judge it from the dashboard's 225 km alone. That is a CLTC test figure and Geely itself notes that actual range varies with driving conditions, climate and usage. Galaxy Geely

Instead, watch whether the indicated range declines reasonably.

For instance:

Start:

100% — 225 km displayed

After driving 50 real km:

~70–80% SOC

may be perfectly reasonable depending on AC use and driving.

What would concern me is something like:

100% → 50% after only 40–50 km

during moderate city driving, especially if energy consumption isn't unusually high.

### Test full-power acceleration

At around 50–80% SOC, find a safe road and accelerate firmly.

Watch for:

- smooth acceleration

- no power limitation warning

- no EV-system warning

- no sudden SOC drop

- no strange noises

- no unexpectedly early gasoline-engine intervention

A damaged or badly degraded battery can sometimes expose itself under higher load even when normal driving appears okay.

### Test regenerative braking

Drive around 60–80 km/h and decelerate normally.

The vehicle's energy-flow display should show power returning toward the battery during regeneration. Geely's hybrid system actively manages electric power flow and energy recovery as part of its hybrid strategy. Galaxy Geely

You may also see SOC increase by 1% after enough downhill/regenerative driving.

### Test it after the car becomes hot

Because your concern is that it sat outside at 37–39°C, I'd specifically test it on a hot day.

After 30–60 minutes of driving:

- park for 10–20 minutes

- restart

- check for battery/system warnings

- charge it if possible

- make sure charging does not repeatedly stop

- watch whether power becomes severely limited

The M7's battery system is designed with thermal protection, and Geely advertises its battery architecture as including cell-, pack-, vehicle- and cloud-level safety management. Galaxy Geely

### The simplest pass/fail test

Without special equipment, I'd be comfortable buying the car if all of these look normal:

| Test | What I'd like to see |
| --- | --- |
| Starts at full charge | ~100% normally |
| EV driving | Smooth, no warnings |
| Actual EV range | Reasonable for conditions |
| SOC decline | Gradual and predictable |
| Charging | Completes normally |
| Charger energy | Consistent with a ~30-kWh-class pack |
| Overnight SOC | Little change |
| Hard acceleration | No sudden power/SOC collapse |
| Regeneration | Works normally |
| Gasoline mode | Engine starts and transitions smoothly |

The best no-tool test is actually the combination of EV range + kWh used to recharge it. It gives you a rough estimate of usable battery capacity.

If you test the M7 and send me just four numbers—

starting SOC, ending SOC, kilometers driven, and kWh used to recharge it—

I can calculate an approximate usable battery capacity for you and tell you whether it looks healthy.