# Smart School Server Design and Installation Guide

Reference architecture for an offline-capable AI platform and student and teacher web applications

Design baseline: 1,000–3,000 users • On premises • High availability • Segmented development environment

This guide gives a complete reference design and installation sequence for a school that needs reliable administrative services, a private AI engine, and a safe platform where students and teachers can build and publish web applications. The recommended result is a small private cloud: three virtualization hosts carry protected services; a dedicated GPU server performs AI inference; storage and backup remain independent; and student workloads are isolated by network policy and permissions.

The design is intentionally vendor-neutral. Equivalent Dell, HPE, Lenovo, or Supermicro hardware may be substituted after confirming firmware, network adapters, GPU support, power, cooling, and local warranty. Final sizing must follow measured concurrency, dataset size, retention, camera workload, and the AI model selected.

# Design decision at a glance

| Area | Recommended decision | Reason |
| --- | --- | --- |
| Core compute | Three identical Proxmox VE nodes | Quorum and maintenance without shutting down all services |
| AI | Dedicated Ubuntu GPU server | Prevents AI jobs from starving school applications |
| Applications | VMs for protected services; Docker for app workloads | Clear isolation with simple operations |
| Student projects | Separate VLAN, Git service, CI runner, quotas and reverse proxy | Limits blast radius of unsafe or broken code |
| Data | Central PostgreSQL service for approved apps; separate test databases | Protects production records |
| Storage | Redundant NAS with snapshots | Shared files and VM data with recoverability |
| Backup | Independent backup server plus offline or off-site copy | A snapshot alone is not a backup |
| Security | Default-deny VLAN firewall rules and MFA for administrators | Student and IoT networks never reach protected data directly |

# 1 Scope and assumptions

Medium school with approximately 1,000–3,000 named users and fewer than 500 concurrent users.

One secure server room with controlled access, cooling, UPS power, grounding, fire detection, and redundant network switching.

Internet may fail; core applications and the knowledge-base AI must continue locally.

The school hosts an LMS, intranet, identity services, databases, file services, monitoring, source control, development workspaces, and dozens of small web applications.

AI is primarily inference and retrieval-augmented generation, not training a foundation model from scratch.

A trained systems administrator owns operations; students do not receive hypervisor, firewall, backup, or production database administration rights.

## Availability targets

| Service class | Examples | Target | Recovery approach |
| --- | --- | --- | --- |
| Critical | Identity, DNS, firewall, student information integration | 99.9% during school hours | HA VMs, configuration backup, tested restore |
| Important | LMS, Git, shared databases, AI gateway | 99.5% | Restart or failover; documented maintenance window |
| Experimental | Student projects and sandboxes | Best effort | Redeploy from Git; no guarantee |
| Archive | Old datasets and project artifacts | Capacity focused | Snapshots plus backup retention |

# 2 Physical and logical architecture

The protected and experimental environments share physical infrastructure only where controls are explicit. The firewall is the policy enforcement point. Management interfaces use a dedicated VLAN and are reachable only from an administrator workstation or VPN.

| Zone / component | Connections | Primary function |
| --- | --- | --- |
| Internet edge | ISP → firewall pair | Filtering, VPN, NAT, intrusion prevention |
| Core switching | Two managed Layer 3 switches | Redundant VLAN trunks and 10/25 Gb server links |
| Compute cluster | PVE01, PVE02, PVE03 | Protected VMs and application VMs |
| AI zone | GPU01 | LLM inference, embeddings, speech and vision services |
| Storage zone | NAS01 | NFS/SMB, datasets, snapshots; optional VM datastore |
| Backup zone | PBS01 | Immutable-minded backup target with separate credentials |
| Development zone | Git, runners, registry, sandbox apps | Student and teacher build and deployment platform |
| Operations zone | Monitoring, logs, bastion, secrets | Administration and audit |

## Architecture diagram

Internet / ISP
      |
 [Firewall HA pair]----[Remote admin VPN]
      |
 [Core switch pair: redundant 10/25 Gb trunks]
      |
      +-- VLAN 10  Management ---- PVE01 / PVE02 / PVE03 / switches / iDRAC-iLO
      +-- VLAN 20  Protected ------ Identity / DNS / LMS / production web / database
      +-- VLAN 30  Development ---- Git / CI runners / registry / student applications
      +-- VLAN 40  AI ------------- GPU01 / AI API / vector service
      +-- VLAN 50  Storage -------- NAS01 / hypervisor storage traffic
      +-- VLAN 60  Backup --------- PBS01 / backup-only paths
      +-- VLAN 70  Teachers ------- staff clients
      +-- VLAN 80  Students ------- student clients
      +-- VLAN 90  IoT and cameras- devices with no lateral access
      +-- VLAN 100 Guest ---------- internet only

Protected services call AI through one authenticated AI gateway.
Student applications cannot connect directly to protected databases or management interfaces.

# 3 Hardware bill of materials

| Qty | Component | Baseline specification | Notes |
| --- | --- | --- | --- |
| 2 | Next-generation firewall appliances | HA capable; 2×10 Gb; support contract or OPNsense appliances | Size to inspected internet throughput, not port speed |
| 2 | Core switches | 24/48 ports; 10/25 Gb SFP+ or SFP28; stacking or MLAG; VLAN and ACL support | Use redundant uplinks |
| 3 | Virtualization hosts | 24–32 physical cores; 384–512 GB ECC RAM; mirrored boot SSD; 4×3.84 TB enterprise NVMe; dual 25 Gb + dual 10 Gb | Identical models simplify failover |
| 1 | GPU AI server | 32+ CPU cores; 256–512 GB ECC RAM; 2× enterprise GPU with 48 GB VRAM each; 8 TB NVMe scratch; dual 25 Gb | Choose GPU after model benchmark |
| 1 | Primary NAS | 12-bay or larger; ECC RAM; ZFS-capable; dual controllers if required; 100 TB usable target | Enterprise HDDs, SSD metadata/cache only if validated |
| 1 | Backup server | 128 GB RAM; large HDD pool; mirrored boot; 10/25 Gb | Separate admin account and network |
| 2 | UPS units | Online double-conversion; network management | A/B power where hardware supports it |
| 1 | Rack and environment | Locked rack, PDUs, sensors, cooling and fire detection | Do not omit facilities work |

Capacity rule: keep at least 25% CPU and memory headroom after losing one virtualization node. Keep storage pools below the vendor’s recommended utilization threshold and reserve capacity for snapshots, rebuilds, and backups.

# 4 Network and IP plan

| VLAN | Name | Example subnet | Who may enter | Important policy |
| --- | --- | --- | --- | --- |
| 10 | Management | 10.10.10.0/24 | Admin VPN and bastion only | No student, teacher, guest, or IoT access |
| 20 | Protected services | 10.10.20.0/23 | Reverse proxy and approved application flows | Databases accept only named application sources |
| 30 | Development | 10.10.30.0/22 | Students and teachers through SSO | No route to management; restricted route to AI gateway |
| 40 | AI | 10.10.40.0/24 | Approved APIs and AI administrators | No direct end-user access to model ports |
| 50 | Storage | 10.10.50.0/24 | Cluster and NAS interfaces only | Non-routed if possible; jumbo frames only end-to-end |
| 60 | Backup | 10.10.60.0/24 | Backup clients to PBS01 | PBS01 initiates no general browsing |
| 70 | Teachers | 10.10.70.0/22 | Managed teacher devices | Access published services, not server admin ports |
| 80 | Students | 10.10.80.0/20 | Managed student devices | Access published apps and approved lab services |
| 90 | IoT and cameras | 10.10.90.0/22 | Registered devices | Internet/vendor endpoints only; no east-west access |
| 100 | Guest | 10.10.100.0/20 | Visitors | Internet only; client isolation |

## Core firewall rules

Default deny between VLANs. Create narrow allow rules by source, destination, port, owner, and expiration date.

Publish HTTP/HTTPS only through reverse proxies or a web application firewall. Never port-forward a database, hypervisor, NAS, SSH, RDP, or model runtime.

Allow the development zone to the AI gateway on HTTPS, not to GPU runtime ports. Apply per-user and per-project rate limits.

Allow backup traffic from registered clients to the backup server. Do not allow backup credentials to be reused on production systems.

Send firewall, SSO, Git, hypervisor, AI gateway, and operating-system logs to a central log service.

# 5 Service placement

| VM or service | Zone | Baseline resources | Deployment |
| --- | --- | --- | --- |
| ID01 / ID02 | Protected | 4 vCPU, 16 GB each | Identity, DNS, NTP integration; two instances |
| PROXY01 / PROXY02 | Protected / DMZ | 4 vCPU, 8 GB each | HAProxy or NGINX; TLS termination |
| DB01 / DB02 | Protected | 16 vCPU, 64–128 GB each | PostgreSQL primary and standby; dedicated volumes |
| APP01 / APP02 | Protected | 8–16 vCPU, 32–64 GB each | LMS and approved school applications |
| GIT01 | Development | 8 vCPU, 32 GB | GitLab or Forgejo; SSO; protected branches |
| RUNNER01+ | Development | 8–16 vCPU, 16–32 GB each | Ephemeral CI runners; no production secrets |
| REGISTRY01 | Development | 4–8 vCPU, 16 GB | Container registry with image scanning |
| DEVAPP01+ | Development | Variable; quotas required | Docker hosts or optional K3s cluster |
| MON01 | Operations | 8 vCPU, 32 GB | Prometheus, Grafana and alert manager |
| LOG01 | Operations | 8–16 vCPU, 32–64 GB | Central logs; retention based on policy |
| AIGW01 | AI | 8 vCPU, 16 GB | Authentication, authorization, safety and quotas |
| GPU01 | AI physical host | As purchased | vLLM or approved inference runtime; embeddings |
| VECTOR01 | Protected or AI | 8 vCPU, 32 GB | pgvector for baseline; Milvus only when scale requires it |

Do not begin with Kubernetes unless the IT team already operates it. A pair of Docker application hosts plus Git-based deployment is adequate for many schools. Add K3s later when multi-node scheduling, self-service namespaces, and formal platform engineering are justified.

# 6 Installation sequence

Use a change log from the first day. Record asset tag, serial number, rack position, firmware, management IP, cable endpoints, VLANs, owner, warranty, and recovery procedure. Never store passwords in this document.

## Phase 0 Prepare and approve

Name an infrastructure owner, security owner, data protection owner, and application owner. Approve a written acceptable-use policy for student hosting and AI.

Inventory applications, data classifications, concurrency, storage growth, retention, recovery time, and recovery point requirements.

Benchmark the intended Korean-capable model and embedding model on candidate GPUs before purchase. Measure tokens per second, concurrent sessions, VRAM, latency, power, and accuracy.

Complete power, cooling, rack, fire, grounding, physical access, cabling, and internet redundancy assessment.

Prepare DNS names, internal certificate authority or ACME approach, IP plan, VLAN plan, firewall matrix, and rollback plan.

## Phase 1 Rack cable and update

Mount UPS, PDUs, switches, firewalls, storage, backup, compute, and GPU equipment using the vendor rail and airflow instructions.

Connect redundant power supplies to different PDUs. Connect server management ports to the management switch ports.

Label both ends of every cable. Use at least two independent uplinks per host; separate management, storage, and workload traffic logically.

Update BIOS, BMC, NIC, storage controller, drive, switch, firewall, and GPU firmware using vendor-supported versions. Export configuration backups.

Set UTC time, NTP, DNS, SNMPv3 or an equivalent secure monitoring protocol, and alert destinations.

## Phase 2 Configure network and firewall

Create VLANs and switch trunks. Configure link aggregation only when the switch and host configuration match.

Create firewall interfaces and DHCP scopes only for client networks. Give servers static addresses or reservations.

Add the default-deny inter-VLAN rule first, then the minimal documented allows. Enable admin VPN with MFA.

Create a dedicated administrator workstation or bastion. Block management interfaces from normal teacher and student clients.

Test every allowed and denied path from representative devices; save the results as the initial security baseline.

## Phase 3 Install the three-node Proxmox cluster

Install the current supported Proxmox VE release from official media on each node. Use mirrored enterprise SSDs for the operating system; do not place the OS on the same pool as bulk VM data.

Assign final management names and IPs before clustering: pve01, pve02, and pve03. Confirm forward and reverse DNS and clock synchronization.

Apply updates, configure the subscription or appropriate repository, create named administrator accounts, enable MFA, and disable routine use of the root account.

Create the cluster on pve01 and join pve02 and pve03. Use the management network for cluster administration and a dedicated low-latency network for corosync when possible.

Configure Linux bridges or VLAN-aware bridges for protected, development, AI, operations, and DMZ workloads. Do not attach VMs directly to the management or backup networks without a documented need.

Choose VM storage: local ZFS replication for a simpler design, shared enterprise NAS/SAN for centralized HA, or Ceph only when the team understands its performance and operational requirements.

Create one noncritical test VM, migrate it between nodes, simulate a node maintenance event, and verify expected HA behavior before installing production services.

Example verification from an administrator shell
pvecm status
pvesh get /cluster/resources
ip -br address
chronyc tracking

Perform cluster creation and node joins from the supported Proxmox UI or the exact commands shown by the installed release. Never copy an old join command from an unrelated system.

## Phase 4 Install storage

Install the selected NAS operating system and current firmware. Create named administrator accounts with MFA if supported.

Build redundant storage pools from enterprise drives. Keep hot spares and replacement drives on site where procurement is slow.

Create separate datasets for protected files, AI datasets, development artifacts, and VM storage. Apply quotas and snapshot schedules independently.

Expose NFS to hypervisor addresses only and SMB to authenticated user groups only. Do not mix anonymous shares with protected data.

Enable encryption where required, save recovery keys offline, configure SMART and pool alerts, and run a scrub and restore test.

## Phase 5 Install the independent backup platform

Install Proxmox Backup Server or an equivalent product on physically separate hardware in the backup VLAN.

Create a dedicated datastore and separate backup identities. Grant the minimum role required to each client.

Schedule nightly VM backups, frequent database-native backups, and configuration exports. Use retention appropriate to policy, such as 7 daily, 5 weekly, and 12 monthly copies.

Replicate or export one encrypted copy to another building, approved cloud object storage, tape, or offline removable media.

Restore a full VM, a database, and an individual file into an isolated network. Record time taken and any missing dependency.

## Phase 6 Build identity DNS and certificates

Integrate with the school identity directory. Use groups for administrators, IT staff, teachers, students, contractors, and service accounts.

Require MFA for administrators and remote access. Use unique service accounts; prohibit shared admin passwords.

Create internal DNS zones and stable service names. Do not make users remember IP addresses.

Deploy a controlled certificate process. Automate renewal and alert before expiration.

Connect Proxmox, Git, monitoring, application portals, VPN, and the AI gateway to SSO where supported.

## Phase 7 Deploy databases and protected applications

Create hardened Ubuntu Server LTS or vendor-supported guest templates using cloud-init. Patch, configure time, logging, endpoint monitoring, and backup before cloning.

Deploy PostgreSQL on dedicated VMs with encrypted connections, per-application roles, separate databases or schemas, and no public listener.

Use a primary and standby database design when uptime requires it. Add connection pooling and monitor replication lag, storage latency, locks, and backup success.

Deploy the LMS and school applications behind redundant reverse proxies. Apply TLS, secure headers, request limits, and a web application firewall where justified.

Perform application-level export backups in addition to VM backups. Document dependencies and a clean-room recovery order.

Representative Ubuntu host baseline
sudo apt update
sudo apt full-upgrade
sudo apt install chrony qemu-guest-agent unattended-upgrades auditd
sudo systemctl enable --now chrony qemu-guest-agent auditd

# Apply SSH, firewall, identity, logging, and monitoring settings through a reviewed
# configuration-management system. Test on a staging VM before broad rollout.

## Phase 8 Build the student and teacher development platform

Deploy GitLab or Forgejo in the development VLAN and connect it to SSO. Enable protected branches, merge review, quotas, and project ownership.

Deploy a private container registry. Scan images and block known critical vulnerabilities according to a documented exception process.

Create ephemeral CI runners with no path to the management VLAN and no standing production credentials. Separate teacher-approved production deployment runners.

Deploy two Docker application hosts for the baseline. Give every project a unique service account, network, hostname, CPU limit, memory limit, storage quota, and log-retention policy.

Put all published applications behind the development reverse proxy. Use wildcard DNS only with a controlled certificate and deployment process.

Provide separate test databases. Allow production database access only after teacher and IT review.

Automatically expire abandoned preview environments and archive source code. Do not promise backup for disposable sandboxes.

Example project delivery flow
Student push → Git review → isolated CI test → image scan → private registry
             → teacher approval → deployment runner → development app host
             → HTTPS reverse proxy → project-name.apps.school.example

## Phase 9 Install the GPU AI platform

Install a vendor-supported Ubuntu Server LTS release on GPU01. Patch BIOS and GPU firmware first, then install the matching NVIDIA driver using the vendor’s supported method.

Install the NVIDIA Container Toolkit from NVIDIA’s official repository and verify that a test container can see the GPUs.

Deploy the selected inference runtime, such as vLLM, in containers. Bind runtime ports only to the AI VLAN.

Deploy an AI gateway in front of the runtime. Require SSO or signed service credentials, apply role-based model access, rate limits, token quotas, request-size limits, safety controls, and audit logs.

For the Korean Q&A system, clean and classify the five-year dataset, remove sensitive data not required for learning, split documents into traceable chunks, create embeddings, and store source metadata with every chunk.

Begin with retrieval-augmented generation. Do not retrain automatically from every new answer. Route new teacher answers through approval, evaluation, and a scheduled re-indexing pipeline.

Evaluate retrieval recall, factuality, Korean language quality, refusal behavior, privacy leakage, latency, and load before launch. Maintain a human escalation path.

AI request path
User → School SSO → AI gateway → policy and quota check → retrieval service
     → approved document chunks → LLM inference → cited response → audit record

Only the gateway is published. Ollama, vLLM, vector databases, and model files remain private.

# 7 Security hardening checklist

| Control | Required implementation | Validation |
| --- | --- | --- |
| Accounts | Named admin accounts, MFA, no shared credentials, quarterly review | Export account and group review evidence |
| Privileges | Role-based access and separate production/development duties | Test a student account against protected targets |
| Patching | Monthly maintenance plus expedited critical fixes | Patch dashboard and exception list |
| Secrets | Vault or protected CI variables; regular rotation | Scan repositories and container images |
| Endpoints | Host firewall, EDR where appropriate, audit logs | Central alert test |
| Network | Default deny, egress filtering, no management from user VLANs | Automated reachability and port test |
| Data | Classification, least access, encryption, retention and deletion | Sample permissions and restore/delete test |
| Applications | TLS, dependency scanning, code review, secure headers | DAST/SAST report and remediation tracking |
| AI | Gateway auth, quotas, prompt and output controls, data minimization | Red-team and privacy evaluation set |
| Backups | Separate credentials, off-site/offline copy, recurring restores | Signed quarterly restore report |
| Facilities | Locked rack, camera/access log, UPS and environmental alerts | Power-failure and temperature alert test |

# 8 Monitoring and operations

Monitor availability, CPU, memory, storage capacity, latency, packet loss, certificates, database replication, backup success, GPU temperature and utilization, model latency, queue depth, and tokens per second.

Alert on symptoms that require action; avoid sending every informational event to staff. Define owner, severity, response time, and escalation for each alert.

Maintain dashboards for infrastructure, protected applications, development quotas, and AI separately.

Run monthly patching, quarterly access reviews and restore tests, semiannual failover exercises, and annual capacity and risk reviews.

Keep architecture diagrams, inventory, vendor contacts, licenses, certificate owners, configuration backups, runbooks, and recovery sequences current.

## Daily weekly and monthly routine

| Frequency | Actions |
| --- | --- |
| Daily | Review critical alerts, failed backups, storage health, security events, and AI gateway errors. |
| Weekly | Review capacity trends, failed CI jobs, exposed student applications, patch advisories, and restore a small sample. |
| Monthly | Patch in staging then production; rotate due secrets; review firewall changes; remove stale accounts and apps. |
| Quarterly | Restore representative systems; review privileges; test UPS alerts; update recovery contacts and runbooks. |
| Annually | Failover exercise, penetration test, AI risk review, warranty and lifecycle plan, full capacity forecast. |

# 9 Commissioning and acceptance tests

| Test | Pass condition |
| --- | --- |
| Node maintenance | One virtualization host is shut down or maintained without losing critical services beyond the approved recovery time. |
| Network isolation | Student, guest, and IoT clients cannot reach management, storage, backup, database, or raw AI runtime ports. |
| Restore | A VM, database, Git project, and individual file restore successfully using written procedures. |
| Internet outage | Core local DNS, identity, LMS or intranet, and approved AI knowledge service remain usable as designed. |
| AI load | Expected concurrent users meet latency and accuracy targets without affecting protected school applications. |
| Application deployment | A student can push, test, scan, receive approval, deploy, view logs, and remove an app without administrator rights. |
| Security logging | A failed admin login, blocked firewall flow, vulnerable image, and AI quota violation create visible events. |
| Power event | UPS alerts arrive; equipment shuts down in the documented order if runtime becomes critical. |

# 10 Migration and rollout plan

| Stage | Duration guide | Exit criterion |
| --- | --- | --- |
| Discovery and sizing | 2–4 weeks | Approved requirements, model benchmark, BOM, IP plan, data classification and recovery targets |
| Facilities and procurement | 4–12 weeks | Rack, power, cooling, warranties, spares and equipment accepted |
| Core build | 2–4 weeks | Network, cluster, storage, backup, identity and monitoring pass tests |
| Platform build | 2–4 weeks | Git, runners, app hosting, database, reverse proxy and AI gateway pass security review |
| Pilot | 4–6 weeks | One department and a small student cohort complete functional and recovery testing |
| Production rollout | 2–4 weeks | Training, support, ownership and rollback are confirmed |
| Optimization | Ongoing | Capacity, model, cost, usability and controls are measured and improved |

# 11 Common mistakes to avoid

Putting school administration, student experiments, cameras, and AI on one flat network.

Buying the most expensive GPU before benchmarking the real model and concurrent load.

Treating RAID, replication, or snapshots as a complete backup strategy.

Running every service directly on the hypervisor or exposing management consoles to the internet.

Giving CI runners permanent production secrets or broad network access.

Letting an AI system continuously learn from unreviewed user input; this creates quality, privacy, and poisoning risks.

Adopting Kubernetes before the staff can securely operate simpler VMs and containers.

Ignoring power, cooling, warranty, spare drives, cable labeling, documentation, and restore drills.

# 12 Information needed for final engineering

This reference design becomes a purchase-ready low-level design only after the school supplies the following:

Student, teacher, staff, and concurrent-user counts; number of campuses and WAN links.

List of web applications, LMS, student information system, directory, camera count, and expected peak traffic.

AI models, languages, context size, target response time, concurrent sessions, vision or speech requirements, and whether any fine-tuning is planned.

Current dataset size, annual growth, retention, privacy classification, and allowed off-site backup locations.

Required uptime, recovery time, recovery point, budget, local vendors, warranty expectations, electricity cost, rack capacity, and cooling capacity.

Existing network, identity provider, domain names, certificates, licenses, IT staffing, and operational skill level.

# Official implementation references

Proxmox VE documentation: https://pve.proxmox.com/pve-docs/

Proxmox Backup Server documentation: https://pbs.proxmox.com/docs/

NVIDIA Container Toolkit install guide: https://docs.nvidia.com/datacenter/cloud-native/container-toolkit/latest/install-guide.html

Docker Engine installation documentation: https://docs.docker.com/engine/install/

K3s documentation for an optional later phase: https://docs.k3s.io/

PostgreSQL documentation: https://www.postgresql.org/docs/

Ubuntu Server documentation: https://documentation.ubuntu.com/server/

CIS Benchmarks: https://www.cisecurity.org/cis-benchmarks

Use the documentation matching the exact installed release. Commands and supported combinations change over time; verify them in a staging environment before production rollout.